Distributed Arbiter Modelling in CCS and STATECRUNCHER - A Comparison

Graham G. Thomason

Appendix to the Thesis “The Design and Construction of a State Machine System

that Handles Nondeterminism”

Department of Computing

School of Electronics and Physical Sciences

University of Surrey

Guildford, Surrey GU2 7XH, UK

July 2004

© Graham G. Thomason 2003-2004

Summary

In this paper we show how a system taken from the CCS literature can be modelled in STATECRUNCHER. An understanding of STATECRUNCHER is assumed, but for the purposes of this paper, most of STATECRUNCHER functionality will not seem strange to anyone familiar with UML dynamic modelling [UML], since that is the basis of the language.

We take a system that is neither too trivial nor too complex to serve as a good case study: the distributed arbiter system as described by Bruns [Bruns p.19]. For the definitive book on CCS by its designer, see [Milner].

STATECRUNCHER was built for the purposes of providing an oracle to state-based tests. It forms part of a tool chain for testing an implementation of a system, i.e. for determining whether the implementation under test behaves according to its specified state behaviour, even when it is nondeterministic. STATECRUNCHER does not generate tests; it co-operates with a test generator in a tool chain.

Table of Contents

1.	Pre-requisite reading	1

2.	The distributed arbiter	2

2.1	Description of the problem	2

2.2	STATECRUNCHER notation and conventions	4

2.3	The single arbiter	6

2.4	Two distributed arbiters	9

2.5	Two distributed arbiters with users	12

2.6	Flattened models	14

3.	The STATECRUNCHER distributed arbiter in CCS	25

4.	Source code of models	29

4.1	Source code of the single distributed arbiter [model t4300]	29

4.2	Source code of  the two distributed arbiters  (John and Mary) [model t4301]	31

4.3	Source code of the distributed arbiter with clients [model t4302]	33

4.4	Source code of single flattened distributed arbiter [model t4310]	36

4.5	Source code of flattened distributed arbiters [model t4311]	37

4.6	Source code of flattened distributed arbiter with clients [model t4312]	39

References	41

# Pre-requisite reading

Two separate appendices are pre-requisite reading to this appendix. They are:

- A Semantic Comparison of STATECRUNCHER and Process Algebras [StCrSemComp]

- A Quick Reference of STATECRUNCHER's Output Format [StCrOutput]

The first also describes differences in terminology between STATECRUNCHER and CCS, and compares their semantics and the way they compose separate state machines into a system. The nondeterministic features of STATECRUNCHER are discussed.

# The distributed arbiter

## Description of the problem

The purpose of this paper is to show how a system taken from the CCS literature can be modelled in STATECRUNCHER. We take a system that is neither too trivial nor too complex to serve as a good case study: The distributed arbiter system as described by Bruns [Bruns p.19]. For the definitive book on CCS by its designer, see [Milner].

The purpose of an arbiter is to manage a serially reusable resource, which we will henceforth just call the resource. If the resource is free, it can be allocated. If the resource is allocated, any other client has to wait (at least) until the resource is released.

Suppose there are two clients for a resource, and these clients run on separate machines in a network, and suppose that communication between the machines is regarded as expensive (probably in terms of time, affecting response times), so communication should be restricted to when it is essential. In this case, the combined arbiter can be constructed out of two single arbiters who share a token which gives the right to allocate the resource. Requests for the token, and replying to the requests by passing the token or saying not-OK will be only be performed if they are essential. So if a client on one machine obtains a resource, and then releases it and requests it again several times, without the other client requesting the resource, no traffic between the machines will ensue. For simplicity, the arbiter does not allow cancelling an unfulfilled request which has been placed for a resource -  once a resource is requested, the client will either get it immediately or must wait for it. (That is how a normal program using a disk server will work, anyway). Figure 1 is a schematic of the distributed system.

Machine 1

Arbiter1

Resource

(e.g. a disk)

Client1

Machine 2

Arbiter2

Client2

- Distributed arbiters and clients for a resource

The CCS notation and state-transition diagram given for a single distributed arbiter are given in [Bruns, p.21] - but we first work in native STATECRUNCHER mode in designing a distributed arbiter. We return to Bruns's model after we have shown the STATECRUNCHER models.

We start with a single arbiter, instances of which run on each distributed machine. We call this arbiter in isolation Me, and its counterpart You, as a kind of template of the arbiter, but later in compositions we will name them John and Mary.

The following colour coding for events and PCOs (points of control and observation) will be used:

- green for a user (i.e. client) event that does not in itself interact with an arbiter, but which may be the event which makes the client want to interact with an arbiter.

- blue for client-arbiter interaction events corresponding to requesting, releasing and acquiring a resource from an arbiter. The requesting and releasing events are ones a user would supply; acquiring is one that would be supplied to a user.

- red for inter-arbiter events

- black for internal events to a single arbiter (if used)

Client-arbiter events: the client of the single arbiter can supply events

MeReqRes	Client tells Me-arbiter that it requests a resource

MeRelRes	Client tells Me-arbiter that it is releasing a resource

MeAcqRes	The Me-arbiter tells the client that the resource has been acquired

The PCO for these events is ClientMePco.

Inter-arbiter events: the events that occur between the arbiter and its counterpart are:

MeReqTok	I request the token

MePass 	I pass the token to you

MeSayNok	I tell you you can't have the token

The PCO for these events is InterArbMePco.

YouReqTok	You request the token

YouPass	You pass the token to me

YouSayNok	You tell me I can't have the token

The PCO for these events is InterArbYouPco.

A STATECRUNCHER model of a single arbiter may or may not make use of internal events. That is perhaps a matter of taste. In this paper, we show two approaches, one with internal events and one without. If the first one seems unnecessary, when you come to it on page 7, skip it by going to page 16. To the outside world, which includes the other arbiter and the client, the behaviour is identical.

The following internal events are used in the relevant models, such as that of Figure 4. They are of local scope, and do not need a Me prefix.

TryTok	I ask another part of myself whether I have the token, and if not, I tell that part of myself to try and get it. The responses may be TryOk (I already have the token), YouSayNok (after asking you for the token, I get a negative response), or AcqTok (I ask you for the token and you pass it to me).

TryOk	I tell another part of myself that the try for the token succeeded (because I already had the token).

AcqTok 	I tell another part of myself that the try for the token succeeded (because I could get it from the You-arbiter).

ResetWant	I tell another part of myself that the You-arbiter need no longer be considered wanting to obtain the token from me.

These events have a null PCO (denoted by [] in STATECRUNCHER).

The arbiter was initially modelled by the author with explicit internal parallelism for

- the state of resource allocation: Idle, Requested, Waiting, Alloc(ated)

- whether the arbiter possesses the token or not: NotHaveTok(en), HaveTok(en)

- whether the arbiter actually needs the token: NotNeedTok(en), NeedTok(en)

- whether the other arbiter wants the token: OtherNotWantTok(en), OtherWantTok(en).

We can flatten (explore) such a model. The Cartesian product of states is potentially 4x2x2x2 = 32, but actually only 6 can exist under proper sequences of events. The flattened model,  which is without internal parallelism is presented later, and as in STATECRUNCHER it produces less output, we will mainly use it. The reader may regard the flattened model as more intuitive from the start or prefer the internally-parallel approach; we will first show the model with internal parallelism.

## STATECRUNCHER notation and conventions

A few details of the model notation and STATECRUNCHER semantics are now explained for convenience. Details are available in the main STATECRUNCHER reports.

STATECRUNCHER's composite states are called clusters and sets. A cluster corresponds to a UML non-concurrent composite state, and to Harel's XOR-states [Harel]. A set corresponds to a UML concurrent composite state, and to Harel's AND-states.

STATECRUNCHER's “after-landing” transition semantics are essential. By this we mean that the actions associated with a transition are carried out after the target states of the transition has been entered, and the pure transition as such is complete. (The alternative is to carry out transition actions in “mid-flight”). In Figure 2, if we are in states state1 and state8, and event alpha is given, the resultant states will be state3 and state9. This is the basis of client-server communication modelling: when the client processes to event alpha, it needs to call the server, which is done by firing event beta. The return event from the server is event gamma. The whole process is carried out without allowing other events to intervene. (Under mid-flight semantics, the transition on gamma would not take place, because its source state, state2, would not be occupied).

state1

state2

state3

state8

state9

gamma

alpha/fire beta

beta/fire gamma

set

client

server

- “After landing” semantics

STATECRUNCHER supports actions on transitions, as in Mealy machines, and actions on exiting or entering states (compare Moore machines). The only kind of action we are concerned with here is the fired event. (In other models, a variable assignment is a common action). The diagrammatic notation used here for on-entry actions is as follows with the  arrow pointing into the state.

s3

fire evt1

- On entry symbol

Transitions can be triggered by internal STATECRUNCHER events, - the exiting and entering of states in a parallel part of the model. Such events are denoted by enter(state) and exit(state).

Conditional transitions have a condition in square brackets. Conditional actions are represented by if (condition) action with optionally else action. Tests can be made for the occupancy of a parallel state using the in(state) function.

States are addressed using scoping operators. In brief, these are:

$x	go up a state then down into x

x%%y	go up until you reach x, then take y

a.x	descend through a and x

::x	start at statechart level and take x

They can be combined into expressions.

Traces in STATECRUNCHER (which are different in concept to those of CCS) are outputs that are observable at black-box level. They are generated in the model by the function trace(expression), which can occur in any STATECRUNCHER action.

The commands to STATECRUNCHER that we will be using are

?- cruncher.	enter the STATECRUNCHER read-process loop

SC:mm			set modelname mode

SC:run t4300	run a model (by modelname, not filename, in this mode)

SC:pe event 	process an event

SC:gc			get configuration

SC:gt			get trace

Reminder for users on how to exit the system

SC:quit 		quit the STATECRUNCHER read-process loop

?- halt.		exit Prolog

## The single arbiter

We now present the single arbiter, and show a session running it. Its STATECRUNCHER source, and that of other models, is given at the end of this paper.

Res

Me

Have

MeReqRes/

fire TryTok

MeRelRes/

if(in($$Me.OtherWant.OtherWantTok))

fire MePass

YouPass/fire AcqTok

MePass/fire ResetWant

OtherNot

WantTok

YouReqTok[in($$Me.Need.NeedTok)]

ResetWant

Need

OtherWant

Other

WantTok

Not

NeedTok

exit($$Me.Res.Idle)

Need

Tok

enter($$Me.Res.Idle)

YouReqTok/fire MePass

YouReqTok/

fire MeSayNok

Not

HaveTok

Have

Tok

Re

quested

Alloc

Idle

Waiting

AcqTok,

TryOk

 fire MeAcqRes

YouSayNok

AcqTok

TryTok/

fire MeReqTok

TryTok/

fire TryOk

- Single arbiter [model t4300]

Session with single arbiter [model t4300]

?- cruncher.

SC:mm

SC:run t4300

...

SC:gc

2    statechart sc

2       set Me [sc] = OCC []  **

2          cluster Res [Me, sc] = OCC []  **

2             leafstate Idle [Res, Me, sc] = OCC []  **

2             leafstate Requested [Res, Me, sc] = VAC []

2             leafstate Waiting [Res, Me, sc] = VAC []

2             leafstate Alloc [Res, Me, sc] = VAC []

2          cluster Have [Me, sc] = OCC []  **

2             leafstate NotHaveTok [Have, Me, sc] = OCC []  **

2             leafstate HaveTok [Have, Me, sc] = VAC []

2          cluster Need [Me, sc] = OCC []  **

2             leafstate NotNeedTok [Need, Me, sc] = OCC []  **

2             leafstate NeedTok [Need, Me, sc] = VAC []

2          cluster OtherWant [Me, sc] = OCC []  **

2             leafstate OtherNotWantTok [OtherWant, Me, sc] = OCC []  **

2             leafstate OtherWantTok [OtherWant, Me, sc] = VAC []

2    TRACE =[]

2    TREV [[MeReqRes, [sc]], 0, [], [ClientMePco, [sc]]]

2    TREV [[TryTok, [sc]], 0, [], []]

2    TREV [[YouPass, [sc]], 0, [], [InterArbYouPco, [sc]]]

2    TREV [[YouReqTok, [sc]], 0, [], [InterArbYouPco, [sc]]]

outworlds=[2]

number of outworlds=1

SC:pe MeReqRes

SC:gc

5    statechart sc

5       set Me [sc] = OCC []  **

5          cluster Res [Me, sc] = OCC []  **

5             leafstate Idle [Res, Me, sc] = VAC []

5             leafstate Requested [Res, Me, sc] = OCC []  **

5             leafstate Waiting [Res, Me, sc] = VAC []

5             leafstate Alloc [Res, Me, sc] = VAC []

5          cluster Have [Me, sc] = OCC []  **

5             leafstate NotHaveTok [Have, Me, sc] = OCC []  **

5             leafstate HaveTok [Have, Me, sc] = VAC []

5          cluster Need [Me, sc] = OCC []  **

5             leafstate NotNeedTok [Need, Me, sc] = VAC []

5             leafstate NeedTok [Need, Me, sc] = OCC []  **

5          cluster OtherWant [Me, sc] = OCC []  **

5             leafstate OtherNotWantTok [OtherWant, Me, sc] = OCC []  **

5             leafstate OtherWantTok [OtherWant, Me, sc] = VAC []

5    TRACE =[]

5    TREV [[AcqTok, [sc]], 0, [], []]

5    TREV [[TryOk, [sc]], 0, [], []]

5    TREV [[YouSayNok, [sc]], 0, [], [InterArbYouPco, [sc]]]

5    TREV [[TryTok, [sc]], 0, [], []]

5    TREV [[YouPass, [sc]], 0, [], [InterArbYouPco, [sc]]]

5    TREV [[YouReqTok, [sc]], 0, [], [InterArbYouPco, [sc]]]

outworlds=[5]

number of outworlds=1

SC:pe YouSayNok

SC:gc

6    statechart sc

6       set Me [sc] = OCC []  **

6          cluster Res [Me, sc] = OCC []  **

6             leafstate Idle [Res, Me, sc] = VAC []

6             leafstate Requested [Res, Me, sc] = VAC []

6             leafstate Waiting [Res, Me, sc] = OCC []  **

6             leafstate Alloc [Res, Me, sc] = VAC []

6          cluster Have [Me, sc] = OCC []  **

6             leafstate NotHaveTok [Have, Me, sc] = OCC []  **

6             leafstate HaveTok [Have, Me, sc] = VAC []

6          cluster Need [Me, sc] = OCC []  **

6             leafstate NotNeedTok [Need, Me, sc] = VAC []

6             leafstate NeedTok [Need, Me, sc] = OCC []  **

6          cluster OtherWant [Me, sc] = OCC []  **

6             leafstate OtherNotWantTok [OtherWant, Me, sc] = OCC []  **

6             leafstate OtherWantTok [OtherWant, Me, sc] = VAC []

6    TRACE =[]

6    TREV [[AcqTok, [sc]], 0, [], []]

6    TREV [[TryTok, [sc]], 0, [], []]

6    TREV [[YouPass, [sc]], 0, [], [InterArbYouPco, [sc]]]

6    TREV [[YouReqTok, [sc]], 0, [], [InterArbYouPco, [sc]]]

outworlds=[6]

number of outworlds=1

SC:pe YouPass

SC:gc

8    statechart sc

8       set Me [sc] = OCC []  **

8          cluster Res [Me, sc] = OCC []  **

8             leafstate Idle [Res, Me, sc] = VAC []

8             leafstate Requested [Res, Me, sc] = VAC []

8             leafstate Waiting [Res, Me, sc] = VAC []

8             leafstate Alloc [Res, Me, sc] = OCC []  **

8          cluster Have [Me, sc] = OCC []  **

8             leafstate NotHaveTok [Have, Me, sc] = VAC []

8             leafstate HaveTok [Have, Me, sc] = OCC []  **

8          cluster Need [Me, sc] = OCC []  **

8             leafstate NotNeedTok [Need, Me, sc] = VAC []

8             leafstate NeedTok [Need, Me, sc] = OCC []  **

8          cluster OtherWant [Me, sc] = OCC []  **

8             leafstate OtherNotWantTok [OtherWant, Me, sc] = OCC []  **

8             leafstate OtherWantTok [OtherWant, Me, sc] = VAC []

8    TRACE =[]

8    TREV [[MeRelRes, [sc]], 0, [], [ClientMePco, [sc]]]

8    TREV [[TryTok, [sc]], 0, [], []]

8    TREV [[MePass, [sc]], 0, [], [InterArbMePco, [sc]]]

8    TREV [[YouReqTok, [sc]], 0, [], [InterArbYouPco, [sc]]]

outworlds=[8]

number of outworlds=1

SC:

## Two distributed arbiters

We now compose a system from two single arbiters, as shown in the following figure:

Mary

Res

Have

Need

OtherWant

MaryReqRes/

fire TryTok

Idle

Re

quested

Waiting

Alloc

Have

Tok

Need

Tok

Other

WantTok

OtherNot

WantTok

Not

NeedTok

Not

HaveTok

JohnSayNok

AcqTok

AcqTok,TryOk

MaryRelRes/

if(in($$Mary.OtherWant.OtherWantTok))

fire MaryPass

TryTok/

fire MaryReqTok

JohnPass/fire AcqTok

MaryPass/fire ResetWant

JohnReqTok/fire MaryPass

exit($$Mary.Res.Idle)

enter($$Mary.Res.Idle)

JohnReqTok[in($$Mary.Need.Tok)]

ResetWant

John

Res

Have

Need

OtherWant

JohnReqRes/

fire TryTok

Idle

Re

quested

Waiting

Alloc

Have

Tok

Need

Tok

Other

WantTok

OtherNot

WantTok

Not

NeedTok

Not

HaveTok

MarySayNok

AcqTok

AcqTok,TryOk

JohnRelRes/

if(in($$John.OtherWant.OtherWantTok))

fire JohnPass

TryTok/

fire JohnReqTok

MaryPass/fire AcqTok

JohnPass/fire ResetWant

MaryReqTok/fire JohnPass

exit($$John.Res.Idle)

enter($$John.Res.Idle)

MaryReqTok[in($$John.Need.Tok)]

ResetWant

Cmp

TryTok/

fire TryOk

TryTok/

fire TryOk

MaryReqTok/

fire

JohnSayNok

JohnReqTok/

fire

MarySayNok

Fire MaryAcqRes

Fire JohnAcqRes

- Two distributed arbiters  (John and Mary) [model t4301]

Session with two distributed arbiters [model t4301]

Output not shown in full, as it is rather lengthy, and equivalent to that of model t4311, shown later.

?- cruncher.

SC:mm

SC:run t4301

...

SC:pe GiveJohnTok

SC:pe MaryReqRes

SC:pe JohnReqRes

SC:gc

19   statechart sc

19      set Cmp [sc] = OCC []  **

19         set John [Cmp, sc] = OCC []  **

19            cluster Res [John, Cmp, sc] = OCC []  **

19               leafstate Idle [Res, John, Cmp, sc] = VAC []

19               leafstate Requested [Res, John, Cmp, sc] = VAC []

19               leafstate Waiting [Res, John, Cmp, sc] = OCC []  **

19               leafstate Alloc [Res, John, Cmp, sc] = VAC []

19            cluster Have [John, Cmp, sc] = OCC []  **

19               leafstate NotHaveTok [Have, John, Cmp, sc] = OCC []  **

19               leafstate HaveTok [Have, John, Cmp, sc] = VAC []

19            cluster Need [John, Cmp, sc] = OCC []  **

19               leafstate NotNeedTok [Need, John, Cmp, sc] = VAC []

19               leafstate NeedTok [Need, John, Cmp, sc] = OCC []  **

19            cluster OtherWant [John, Cmp, sc] = OCC []  **

19               leafstate OtherNotWantTok [OtherWant, John, Cmp, sc] = OCC []  **

19               leafstate OtherWantTok [OtherWant, John, Cmp, sc] = VAC []

19         set Mary [Cmp, sc] = OCC []  **

19            cluster Res [Mary, Cmp, sc] = OCC []  **

19               leafstate Idle [Res, Mary, Cmp, sc] = VAC []

19               leafstate Requested [Res, Mary, Cmp, sc] = VAC []

19               leafstate Waiting [Res, Mary, Cmp, sc] = VAC []

19               leafstate Alloc [Res, Mary, Cmp, sc] = OCC []  **

19            cluster Have [Mary, Cmp, sc] = OCC []  **

19               leafstate NotHaveTok [Have, Mary, Cmp, sc] = VAC []

19               leafstate HaveTok [Have, Mary, Cmp, sc] = OCC []  **

19            cluster Need [Mary, Cmp, sc] = OCC []  **

19               leafstate NotNeedTok [Need, Mary, Cmp, sc] = VAC []

19               leafstate NeedTok [Need, Mary, Cmp, sc] = OCC []  **

19            cluster OtherWant [Mary, Cmp, sc] = OCC []  **

19               leafstate OtherNotWantTok [OtherWant, Mary, Cmp, sc] = VAC []

19               leafstate OtherWantTok [OtherWant, Mary, Cmp, sc] = OCC []  **

19   TRACE =[]

19   TREV [[AcqTok, [John, Cmp, sc]], 0, [], []]

19   TREV [[TryTok, [John, Cmp, sc]], 0, [], []]

19   TREV [[MaryPass, [Cmp, sc]], 0, [], [InterArbMaryPco, [Cmp, sc]]]

19   TREV [[MaryReqTok, [Cmp, sc]], 0, [], [InterArbMaryPco, [Cmp, sc]]]

19   TREV [[MaryRelRes, [sc]], 0, [], [ClientMaryPco, [sc]]]

19   TREV [[TryTok, [Mary, Cmp, sc]], 0, [], []]

19   TREV [[JohnReqTok, [Cmp, sc]], 0, [], [InterArbJohnPco, [Cmp, sc]]]

19   TREV [[ResetWant, [Mary, Cmp, sc]], 0, [], []]

19   TREV [[GiveJohnTok, [sc]], 0, [], []]

outworlds=[19]

number of outworlds=1

SC:pe MaryRelRes

...

## Two distributed arbiters with users

The following figure shows users (clients) composed into a system with two distributed arbiters:

Cmp

SysJ

John

alpha/

fire JohnReqRes

NotHave

Res

Waiting

UserJ

Have

Res

beta/

fire JohnRelRes

JohnAcqRes

JohnReqRes

/ fire JohnAcqRes

Details of internal state behaviour unknown

SysM

Mary

gamma/

fire MaryReqRes

NotHave

Res

Waiting

UserM

Have

Res

delta/

fire MaryRelRes

MaryAcqRes

MaryReqRes

/ fire MaryAcqRes

Details of internal state behaviour unknown

JohnRelRes

MaryRelRes

 trace(MAcq)

 trace(MRel)

 trace(JAcq)

 trace(JRel)

- Two distributed arbiters with users (black box view) [model t4302]

In this model the clients need the resource when they process events alpha and gamma. The clients release the resource on events beta and delta. We drive the system using these events only.

Session with two distributed arbiters with users

Reminder: the TRACE is read from right to left.

Read

JAcq as the user of arbiter John acquired the resource

JRel as the user of arbiter John released the resource

MAcq as the user of arbiter Mary acquired the resource

MRel as the user of arbiter Mary released the resource

?- cruncher.

SC:mm

SC:run t4302

...

SC:gt

2    TRACE =[]

SC:pe GiveJohnTok

SC:gt

3    TRACE =[]

SC:pe alpha

SC:gt

10   TRACE =[JAcq]

SC:pe gamma

SC:gt

17   TRACE =[JAcq]

SC:pe beta

SC:gt

36   TRACE =[MAcq, JRel, JAcq]

SC:pe delta

SC:gt

40   TRACE =[MRel, MAcq, JRel, JAcq]

SC:gc

40   statechart sc

40      set Cmp [sc] = OCC []  **

40         set SysJ [Cmp, sc] = OCC []  **

40            set John [SysJ, Cmp, sc] = OCC []  **

40               cluster Res [John, SysJ, Cmp, sc] = OCC []  **

40                  leafstate Idle [Res, John, SysJ, Cmp, sc] = OCC []  **

40                  leafstate Requested [Res, John, SysJ, Cmp, sc] = VAC []

40                  leafstate Waiting [Res, John, SysJ, Cmp, sc] = VAC []

40                  leafstate Alloc [Res, John, SysJ, Cmp, sc] = VAC []

40               cluster Have [John, SysJ, Cmp, sc] = OCC []  **

40                  leafstate NotHaveTok [Have, John, SysJ, Cmp, sc] = OCC []  **

40                  leafstate HaveTok [Have, John, SysJ, Cmp, sc] = VAC []

40               cluster Need [John, SysJ, Cmp, sc] = OCC []  **

40                  leafstate NotNeedTok [Need, John, SysJ, Cmp, sc] = OCC []  **

40                  leafstate NeedTok [Need, John, SysJ, Cmp, sc] = VAC []

40               cluster OtherWant [John, SysJ, Cmp, sc] = OCC []  **

40                  leafstate OtherNotWantTok [OtherWant, John, SysJ, Cmp, sc] = OCC []  **

40                  leafstate OtherWantTok [OtherWant, John, SysJ, Cmp, sc] = VAC []

40            cluster UserJ [SysJ, Cmp, sc] = OCC []  **

40               leafstate NotHaveRes [UserJ, SysJ, Cmp, sc] = OCC []  **

40               leafstate Waiting [UserJ, SysJ, Cmp, sc] = VAC []

40               leafstate HaveRes [UserJ, SysJ, Cmp, sc] = VAC []

40         set SysM [Cmp, sc] = OCC []  **

40            set Mary [SysM, Cmp, sc] = OCC []  **

40               cluster Res [Mary, SysM, Cmp, sc] = OCC []  **

40                  leafstate Idle [Res, Mary, SysM, Cmp, sc] = OCC []  **

40                  leafstate Requested [Res, Mary, SysM, Cmp, sc] = VAC []

40                  leafstate Waiting [Res, Mary, SysM, Cmp, sc] = VAC []

40                  leafstate Alloc [Res, Mary, SysM, Cmp, sc] = VAC []

40               cluster Have [Mary, SysM, Cmp, sc] = OCC []  **

40                  leafstate NotHaveTok [Have, Mary, SysM, Cmp, sc] = VAC []

40                  leafstate HaveTok [Have, Mary, SysM, Cmp, sc] = OCC []  **

40               cluster Need [Mary, SysM, Cmp, sc] = OCC []  **

40                  leafstate NotNeedTok [Need, Mary, SysM, Cmp, sc] = OCC []  **

40                  leafstate NeedTok [Need, Mary, SysM, Cmp, sc] = VAC []

40               cluster OtherWant [Mary, SysM, Cmp, sc] = OCC []  **

40                  leafstate OtherNotWantTok [OtherWant, Mary, SysM, Cmp, sc] = OCC []  **

40                  leafstate OtherWantTok [OtherWant, Mary, SysM, Cmp, sc] = VAC []

40            cluster UserM [SysM, Cmp, sc] = OCC []  **

40               leafstate NotHaveRes [UserM, SysM, Cmp, sc] = OCC []  **

40               leafstate Waiting [UserM, SysM, Cmp, sc] = VAC []

40               leafstate HaveRes [UserM, SysM, Cmp, sc] = VAC []

40   TRACE =[MRel, MAcq, JRel, JAcq]

40   TREV [[JohnReqRes, [sc]], 0, [], [ClientJohnPco, [sc]]]

40   TREV [[TryTok, [John, SysJ, Cmp, sc]], 0, [], []]

40   TREV [[MaryPass, [Cmp, sc]], 0, [], [InterArbMaryPco, [Cmp, sc]]]

40   TREV [[MaryReqTok, [Cmp, sc]], 0, [], [InterArbMaryPco, [Cmp, sc]]]

40   TREV [[alpha, [sc]], 0, [], [UserJPco, [sc]]]

40   TREV [[MaryReqRes, [sc]], 0, [], [ClientMaryPco, [sc]]]

40   TREV [[TryTok, [Mary, SysM, Cmp, sc]], 0, [], []]

40   TREV [[JohnReqTok, [Cmp, sc]], 0, [], [InterArbJohnPco, [Cmp, sc]]]

40   TREV [[gamma, [sc]], 0, [], [UserMPco, [sc]]]

40   TREV [[GiveJohnTok, [sc]], 0, [], []]

outworlds=[40]

number of outworlds=1

SC:

## Flattened models

The diagrams following show an alternative model to the distributed arbiter, using just a cluster. We first show the flattened states of the model of Figure 4, then a new model based on the flattened states.

Res

Me

Have

MeReqRes/

fire TryTok

MeRelRes/

if in($$Me.OtherWant.OtherWantTok) fire MePass

YouPass/fire AcqTok

MePass/fire ResetWant

OtherNot

WantTok

YouReqTok[in $$Me.Need.NeedTok]

ResetWant

Need

OtherWant

Other

WantTok

Not

NeedTok

exit $$Me.Res.Idle

Need

Tok

enter $$Me.Res.Idle

YouReqTok/fire MePass

YouReqTok/

fire MeSayNok

Not

HaveTok

Have

Tok

Re

quested

Alloc

Idle

Waiting

AcqTok,

TryOk

MeNok

AcqTok

TryTok/fire MeReqTok

TryTok/

fire TryOk

Res

Me

Have

MeReqRes/

fire TryTok

MeRelRes/

if in($$Me.OtherWant.OtherWantTok) fire MePass

YouPass/fire AcqTok

MePass/fire ResetWant

OtherNot

WantTok

YouReqTok[in $$Me.Need.NeedTok]

ResetWant

Need

OtherWant

Other

WantTok

Not

NeedTok

exit $$Me.Res.Idle

Need

Tok

enter $$Me.Res.Idle

YouReqTok/fire MePass

Not

HaveTok

Have

Tok

Re

quested

Alloc

Idle

Waiting

AcqTok,

TryOk

MeNok

AcqTok

TryTok/fire MeReqTok

TryTok/

fire TryOk

MeReqRes/fire MeReqTok

YouSayNok

Res

Me

Have

MeReqRes/

fire TryTok

MeRelRes/

if in($$Me.OtherWant.OtherWantTok) fire MePass

YouPass/fire AcqTok

MePass/fire ResetWant

OtherNot

WantTok

YouReqTok[in $$Me.Need.NeedTok]

ResetWant

Need

OtherWant

Other

WantTok

Not

NeedTok

exit $$Me.Res.Idle

Need

Tok

enter $$Me.Res.Idle

YouReqTok/fire MePass

Not

HaveTok

Have

Tok

Re

quested

Alloc

Idle

Waiting

AcqTok,

TryOk

MeNok

AcqTok

TryTok/fire MeReqTok

TryTok/

fire TryOk

YouPass

Res

Me

Have

MeReqRes/

fire TryTok

MeRelRes/

if in($$Me.OtherWant.OtherWantTok) fire MePass

YouPass/fire AcqTok

MePass/fire ResetWant

OtherNot

WantTok

YouReqTok[in $$Me.Need.NeedTok]

ResetWant

Need

OtherWant

Other

WantTok

Not

NeedTok

exit $$Me.Res.Idle

Need

Tok

enter $$Me.Res.Idle

YouReqTok/fire MePass

Not

HaveTok

Have

Tok

Re

quested

Alloc

Idle

Waiting

AcqTok,

TryOk

MeNok

AcqTok

TryTok/fire MeReqTok

TryTok/

fire TryOk

Res

Me

Have

MeReqRes/

fire TryTok

MeRelRes/

if in($$Me.OtherWant.OtherWantTok) fire MePass

YouPass/fire AcqTok

MePass/fire ResetWant

OtherNot

WantTok

YouReqTok[in $$Me.Need.NeedTok]

ResetWant

Need

OtherWant

Other

WantTok

Not

NeedTok

exit $$Me.Res.Idle

Need

Tok

enter $$Me.Res.Idle

YouReqTok/fire MePass

Not

HaveTok

Have

Tok

Re

quested

Alloc

Idle

Waiting

AcqTok,

TryOk

MeNok

AcqTok

TryTok/fire MeReqTok

TryTok/

fire TryOk

Res

Me

Have

MeReqRes/

fire TryTok

MeRelRes/

if in($$Me.OtherWant.OtherWantTok) fire MePass

YouPass/fire AcqTok

MePass/fire ResetWant

OtherNot

WantTok

YouReqTok[in $$Me.Need.NeedTok]

ResetWant

Need

OtherWant

Other

WantTok

Not

NeedTok

exit $$Me.Res.Idle

Need

Tok

enter $$Me.Res.Idle

YouReqTok/fire MePass

Not

HaveTok

Have

Tok

Re

quested

Alloc

Idle

Waiting

AcqTok,

TryOk

MeNok

AcqTok

TryTok/fire MeReqTok

TryTok/

fire TryOk

YouReqTok /fire MePass

YouPass

MeRelRes

YouReqTok /fire MeSayNok

MeReqRes

YouReqTok/

fire MeSayNok

YouReqTok/

fire MeSayNok

YouReqTok/

fire MeSayNok

YouReqTok/

fire MeSayNok

YouReqTok/

fire MeSayNok

MeRelRes /fire MePass

- The model flattened (explored, unfolded)

m1_

Idle

NoTok

m2_

Reqd

Tok

m4_

Alloc

Plain

m6_

Alloc

OtherWant

m5_

Idle

WithTok

m3_

Waiting

MeReqRes

/fire MeReqTok

Me

YouPass

YouSayNok

YouPass

YouReqTok

/fire

MeSayNok

MeRelRes

MeReqRes

MeRelRes

/fire MePass

YouReqTok

/fire

MePass

 fire MeAcqRes

- Single flattened distributed arbiter [model t4310]

Session with single flattened arbiter

| ?- cruncher.

SC:|: mm

SC:|: run t4310

...

SC:|: gc

2    statechart sc

2       cluster Me [sc] = OCC []  **

2          leafstate m1_IdleNoTok [Me,sc] = OCC []  **

2          leafstate m2_ReqdTok [Me,sc] = VAC []

2          leafstate m3_Waiting [Me,sc] = VAC []

2          leafstate m4_AllocPlain [Me,sc] = VAC []

2          leafstate m5_IdleWithTok [Me,sc] = VAC []

2          leafstate m6_AllocOtherWant [Me,sc] = VAC []

2    TRACE =[]

2    TREV [[MeReqRes,[sc]],0,[],[ClientMePco,[sc]]]

outworlds=[2]

number of outworlds=1

SC:|: pe JohnReqRes

SC:|: gc

2    statechart sc

2       cluster Me [sc] = OCC []  **

2          leafstate m1_IdleNoTok [Me,sc] = OCC []  **

2          leafstate m2_ReqdTok [Me,sc] = VAC []

2          leafstate m3_Waiting [Me,sc] = VAC []

2          leafstate m4_AllocPlain [Me,sc] = VAC []

2          leafstate m5_IdleWithTok [Me,sc] = VAC []

2          leafstate m6_AllocOtherWant [Me,sc] = VAC []

2    TRACE =[]

2    TREV [[MeReqRes,[sc]],0,[],[ClientMePco,[sc]]]

outworlds=[2]

number of outworlds=1

SC:|: pe MarySayNok

SC:|: gc

2    statechart sc

2       cluster Me [sc] = OCC []  **

2          leafstate m1_IdleNoTok [Me,sc] = OCC []  **

2          leafstate m2_ReqdTok [Me,sc] = VAC []

2          leafstate m3_Waiting [Me,sc] = VAC []

2          leafstate m4_AllocPlain [Me,sc] = VAC []

2          leafstate m5_IdleWithTok [Me,sc] = VAC []

2          leafstate m6_AllocOtherWant [Me,sc] = VAC []

2    TRACE =[]

2    TREV [[MeReqRes,[sc]],0,[],[ClientMePco,[sc]]]

outworlds=[2]

number of outworlds=1

SC:|: pe MaryPass

SC:|: gc

2    statechart sc

2       cluster Me [sc] = OCC []  **

2          leafstate m1_IdleNoTok [Me,sc] = OCC []  **

2          leafstate m2_ReqdTok [Me,sc] = VAC []

2          leafstate m3_Waiting [Me,sc] = VAC []

2          leafstate m4_AllocPlain [Me,sc] = VAC []

2          leafstate m5_IdleWithTok [Me,sc] = VAC []

2          leafstate m6_AllocOtherWant [Me,sc] = VAC []

2    TRACE =[]

2    TREV [[MeReqRes,[sc]],0,[],[ClientMePco,[sc]]]

outworlds=[2]

number of outworlds=1

SC:|:

m6_Alloc

Other

Want

m4_

Alloc

Plain

m5_

Idle

WithTok

m3_

Waiting

m1_

Idle

NoTok

m2_

Reqd

Tok

MaryPass

MaryReqTok

/fire

JohnSayNok

JohnRelRes

/fire JohnPass

JohnReqRes

JohnRelRes

MaryPass

MarySayNok

JohnReqRes

/fire JohnReqTok

MaryReqTok

/fire

JohnPass

m6_Alloc

Other

Want

m4_

Alloc

Plain

m5_

Idle

WithTok

m3_

Waiting

m1_

Idle

NoTok

m2_

Reqd

Tok

JohnPass

JohnReqTok

/fire

MarySayNok

MaryRelRes

/fire MaryPass

MaryReqRes

MaryRelRes

JohnPass

JohnSayNok

MaryReqRes

/fire MaryReqTok

JohnReqTok

/fire

MaryPass

Cmp

John

Mary

 fire JohnAcqRes

 fire MaryAcqRes

- Two flattened distributed arbiters [model t4311]

Session with two flattened distributed arbiters

| ?- cruncher.

SC:|: mm

SC:|: run t4311

...

SC:|: pe GiveJohnTok

SC:|: gc

3    statechart sc

3       set Cmp [sc] = OCC []  **

3          cluster John [Cmp,sc] = OCC []  **

3             leafstate m1_IdleNoTok [John,Cmp,sc] = VAC []

3             leafstate m2_ReqdTok [John,Cmp,sc] = VAC []

3             leafstate m3_Waiting [John,Cmp,sc] = VAC []

3             leafstate m4_AllocPlain [John,Cmp,sc] = VAC []

3             leafstate m5_IdleWithTok [John,Cmp,sc] = OCC []  **

3             leafstate m6_AllocOtherWant [John,Cmp,sc] = VAC []

3          cluster Mary [Cmp,sc] = OCC []  **

3             leafstate m1_IdleNoTok [Mary,Cmp,sc] = OCC []  **

3             leafstate m2_ReqdTok [Mary,Cmp,sc] = VAC []

3             leafstate m3_Waiting [Mary,Cmp,sc] = VAC []

3             leafstate m4_AllocPlain [Mary,Cmp,sc] = VAC []

3             leafstate m5_IdleWithTok [Mary,Cmp,sc] = VAC []

3             leafstate m6_AllocOtherWant [Mary,Cmp,sc] = VAC []

3    TRACE =[]

3    TREV [[MaryReqTok,[Cmp,sc]],0,[],[InterArbMaryPco,[Cmp,sc]]]

3    TREV [[JohnReqRes,[sc]],0,[],[ClientJohnPco,[sc]]]

3    TREV [[MaryReqRes,[sc]],0,[],[ClientMaryPco,[sc]]]

3    TREV [[GiveJohnTok,[sc]],0,[],[]]

outworlds=[3]

number of outworlds=1

SC:|: pe MaryReqRes

SC:|: gc

6    statechart sc

6       set Cmp [sc] = OCC []  **

6          cluster John [Cmp,sc] = OCC []  **

6             leafstate m1_IdleNoTok [John,Cmp,sc] = OCC []  **

6             leafstate m2_ReqdTok [John,Cmp,sc] = VAC []

6             leafstate m3_Waiting [John,Cmp,sc] = VAC []

6             leafstate m4_AllocPlain [John,Cmp,sc] = VAC []

6             leafstate m5_IdleWithTok [John,Cmp,sc] = VAC []

6             leafstate m6_AllocOtherWant [John,Cmp,sc] = VAC []

6          cluster Mary [Cmp,sc] = OCC []  **

6             leafstate m1_IdleNoTok [Mary,Cmp,sc] = VAC []

6             leafstate m2_ReqdTok [Mary,Cmp,sc] = VAC []

6             leafstate m3_Waiting [Mary,Cmp,sc] = VAC []

6             leafstate m4_AllocPlain [Mary,Cmp,sc] = OCC []  **

6             leafstate m5_IdleWithTok [Mary,Cmp,sc] = VAC []

6             leafstate m6_AllocOtherWant [Mary,Cmp,sc] = VAC []

6    TRACE =[]

6    TREV [[JohnReqRes,[sc]],0,[],[ClientJohnPco,[sc]]]

6    TREV [[MaryRelRes,[sc]],0,[],[ClientMaryPco,[sc]]]

6    TREV [[JohnReqTok,[Cmp,sc]],0,[],[InterArbJohnPco,[Cmp,sc]]]

6    TREV [[GiveJohnTok,[sc]],0,[],[]]

outworlds=[6]

number of outworlds=1

SC:|: pe JohnReqRes

SC:|: gc

9    statechart sc

9       set Cmp [sc] = OCC []  **

9          cluster John [Cmp,sc] = OCC []  **

9             leafstate m1_IdleNoTok [John,Cmp,sc] = VAC []

9             leafstate m2_ReqdTok [John,Cmp,sc] = VAC []

9             leafstate m3_Waiting [John,Cmp,sc] = OCC []  **

9             leafstate m4_AllocPlain [John,Cmp,sc] = VAC []

9             leafstate m5_IdleWithTok [John,Cmp,sc] = VAC []

9             leafstate m6_AllocOtherWant [John,Cmp,sc] = VAC []

9          cluster Mary [Cmp,sc] = OCC []  **

9             leafstate m1_IdleNoTok [Mary,Cmp,sc] = VAC []

9             leafstate m2_ReqdTok [Mary,Cmp,sc] = VAC []

9             leafstate m3_Waiting [Mary,Cmp,sc] = VAC []

9             leafstate m4_AllocPlain [Mary,Cmp,sc] = VAC []

9             leafstate m5_IdleWithTok [Mary,Cmp,sc] = VAC []

9             leafstate m6_AllocOtherWant [Mary,Cmp,sc] = OCC []  **

9    TRACE =[]

9    TREV [[MaryPass,[Cmp,sc]],0,[],[InterArbMaryPco,[Cmp,sc]]]

9    TREV [[MaryRelRes,[sc]],0,[],[ClientMaryPco,[sc]]]

9    TREV [[GiveJohnTok,[sc]],0,[],[]]

outworlds=[9]

number of outworlds=1

SC:|: pe MaryRelRes

SC:|: gc

11   statechart sc

11      set Cmp [sc] = OCC []  **

11         cluster John [Cmp,sc] = OCC []  **

11            leafstate m1_IdleNoTok [John,Cmp,sc] = VAC []

11            leafstate m2_ReqdTok [John,Cmp,sc] = VAC []

11            leafstate m3_Waiting [John,Cmp,sc] = VAC []

11            leafstate m4_AllocPlain [John,Cmp,sc] = OCC []  **

11            leafstate m5_IdleWithTok [John,Cmp,sc] = VAC []

11            leafstate m6_AllocOtherWant [John,Cmp,sc] = VAC []

11         cluster Mary [Cmp,sc] = OCC []  **

11            leafstate m1_IdleNoTok [Mary,Cmp,sc] = OCC []  **

11            leafstate m2_ReqdTok [Mary,Cmp,sc] = VAC []

11            leafstate m3_Waiting [Mary,Cmp,sc] = VAC []

11            leafstate m4_AllocPlain [Mary,Cmp,sc] = VAC []

11            leafstate m5_IdleWithTok [Mary,Cmp,sc] = VAC []

11            leafstate m6_AllocOtherWant [Mary,Cmp,sc] = VAC []

11   TRACE =[]

11   TREV [[JohnRelRes,[sc]],0,[],[ClientJohnPco,[sc]]]

11   TREV [[MaryReqTok,[Cmp,sc]],0,[],[InterArbMaryPco,[Cmp,sc]]]

11   TREV [[MaryReqRes,[sc]],0,[],[ClientMaryPco,[sc]]]

11   TREV [[GiveJohnTok,[sc]],0,[],[]]

outworlds=[11]

number of outworlds=1

SC:|:

Cmp

SysJ

John

alpha/

fire JohnReqRes

NotHave

Res

Waiting

UserJ

Have

Res

beta/

fire JohnRelRes

JohnAcqRes

JohnReqRes

/ fire JohnAcqRes

Details of internal state behaviour unknown

SysM

Mary

gamma/

fire MaryReqRes

NotHave

Res

Waiting

UserM

Have

Res

delta/

fire MaryRelRes

MaryAcqRes

MaryReqRes

/ fire MaryAcqRes

Details of internal state behaviour unknown

JohnRelRes

MaryRelRes

 trace(MAcq)

 trace(MRel)

 trace(JAcq)

 trace(JRel)

Flattened model and session with users - Repeat of Figure 6, [model t4312]

| ?- cruncher.

SC:|: mm

SC:|: run t4312

...

SC:|: pe GiveJohnTok

SC:|: gc

3    statechart sc

3       set Cmp [sc] = OCC []  **

3          set SysJ [Cmp,sc] = OCC []  **

3             cluster John [SysJ,Cmp,sc] = OCC []  **

3                leafstate m1_IdleNoTok [John,SysJ,Cmp,sc] = VAC []

3                leafstate m2_ReqdTok [John,SysJ,Cmp,sc] = VAC []

3                leafstate m3_Waiting [John,SysJ,Cmp,sc] = VAC []

3                leafstate m4_AllocPlain [John,SysJ,Cmp,sc] = VAC []

3                leafstate m5_IdleWithTok [John,SysJ,Cmp,sc] = OCC []  **

3                leafstate m6_AllocOtherWant [John,SysJ,Cmp,sc] = VAC []

3             cluster UserJ [SysJ,Cmp,sc] = OCC []  **

3                leafstate NotHaveRes [UserJ,SysJ,Cmp,sc] = OCC []  **

3                leafstate Waiting [UserJ,SysJ,Cmp,sc] = VAC []

3                leafstate HaveRes [UserJ,SysJ,Cmp,sc] = VAC []

3          set SysM [Cmp,sc] = OCC []  **

3             cluster Mary [SysM,Cmp,sc] = OCC []  **

3                leafstate m1_IdleNoTok [Mary,SysM,Cmp,sc] = OCC []  **

3                leafstate m2_ReqdTok [Mary,SysM,Cmp,sc] = VAC []

3                leafstate m3_Waiting [Mary,SysM,Cmp,sc] = VAC []

3                leafstate m4_AllocPlain [Mary,SysM,Cmp,sc] = VAC []

3                leafstate m5_IdleWithTok [Mary,SysM,Cmp,sc] = VAC []

3                leafstate m6_AllocOtherWant [Mary,SysM,Cmp,sc] = VAC []

3             cluster UserM [SysM,Cmp,sc] = OCC []  **

3                leafstate NotHaveRes [UserM,SysM,Cmp,sc] = OCC []  **

3                leafstate Waiting [UserM,SysM,Cmp,sc] = VAC []

3                leafstate HaveRes [UserM,SysM,Cmp,sc] = VAC []

3    TRACE =[]

3    TREV [[MaryReqTok,[Cmp,sc]],0,[],[InterArbMaryPco,[Cmp,sc]]]

3    TREV [[JohnReqRes,[sc]],0,[],[ClientJohnPco,[sc]]]

3    TREV [[alpha,[sc]],0,[],[UserJPco,[sc]]]

3    TREV [[MaryReqRes,[sc]],0,[],[ClientMaryPco,[sc]]]

3    TREV [[gamma,[sc]],0,[],[UserMPco,[sc]]]

3    TREV [[GiveJohnTok,[sc]],0,[],[]]

outworlds=[3]

number of outworlds=1

SC:|: pe alpha

SC:|: gc

7    statechart sc

7       set Cmp [sc] = OCC []  **

7          set SysJ [Cmp,sc] = OCC []  **

7             cluster John [SysJ,Cmp,sc] = OCC []  **

7                leafstate m1_IdleNoTok [John,SysJ,Cmp,sc] = VAC []

7                leafstate m2_ReqdTok [John,SysJ,Cmp,sc] = VAC []

7                leafstate m3_Waiting [John,SysJ,Cmp,sc] = VAC []

7                leafstate m4_AllocPlain [John,SysJ,Cmp,sc] = OCC []  **

7                leafstate m5_IdleWithTok [John,SysJ,Cmp,sc] = VAC []

7                leafstate m6_AllocOtherWant [John,SysJ,Cmp,sc] = VAC []

7             cluster UserJ [SysJ,Cmp,sc] = OCC []  **

7                leafstate NotHaveRes [UserJ,SysJ,Cmp,sc] = VAC []

7                leafstate Waiting [UserJ,SysJ,Cmp,sc] = VAC []

7                leafstate HaveRes [UserJ,SysJ,Cmp,sc] = OCC []  **

7          set SysM [Cmp,sc] = OCC []  **

7             cluster Mary [SysM,Cmp,sc] = OCC []  **

7                leafstate m1_IdleNoTok [Mary,SysM,Cmp,sc] = OCC []  **

7                leafstate m2_ReqdTok [Mary,SysM,Cmp,sc] = VAC []

7                leafstate m3_Waiting [Mary,SysM,Cmp,sc] = VAC []

7                leafstate m4_AllocPlain [Mary,SysM,Cmp,sc] = VAC []

7                leafstate m5_IdleWithTok [Mary,SysM,Cmp,sc] = VAC []

7                leafstate m6_AllocOtherWant [Mary,SysM,Cmp,sc] = VAC []

7             cluster UserM [SysM,Cmp,sc] = OCC []  **

7                leafstate NotHaveRes [UserM,SysM,Cmp,sc] = OCC []  **

7                leafstate Waiting [UserM,SysM,Cmp,sc] = VAC []

7                leafstate HaveRes [UserM,SysM,Cmp,sc] = VAC []

7    TRACE =[JAcq]

7    TREV [[JohnRelRes,[sc]],0,[],[ClientJohnPco,[sc]]]

7    TREV [[MaryReqTok,[Cmp,sc]],0,[],[InterArbMaryPco,[Cmp,sc]]]

7    TREV [[beta,[sc]],0,[],[UserJPco,[sc]]]

7    TREV [[MaryReqRes,[sc]],0,[],[ClientMaryPco,[sc]]]

7    TREV [[gamma,[sc]],0,[],[UserMPco,[sc]]]

7    TREV [[GiveJohnTok,[sc]],0,[],[]]

outworlds=[7]

number of outworlds=1

SC:|: pe gamma

SC:|: gc

11   statechart sc

11      set Cmp [sc] = OCC []  **

11         set SysJ [Cmp,sc] = OCC []  **

11            cluster John [SysJ,Cmp,sc] = OCC []  **

11               leafstate m1_IdleNoTok [John,SysJ,Cmp,sc] = VAC []

11               leafstate m2_ReqdTok [John,SysJ,Cmp,sc] = VAC []

11               leafstate m3_Waiting [John,SysJ,Cmp,sc] = VAC []

11               leafstate m4_AllocPlain [John,SysJ,Cmp,sc] = VAC []

11               leafstate m5_IdleWithTok [John,SysJ,Cmp,sc] = VAC []

11               leafstate m6_AllocOtherWant [John,SysJ,Cmp,sc] = OCC []  **

11            cluster UserJ [SysJ,Cmp,sc] = OCC []  **

11               leafstate NotHaveRes [UserJ,SysJ,Cmp,sc] = VAC []

11               leafstate Waiting [UserJ,SysJ,Cmp,sc] = VAC []

11               leafstate HaveRes [UserJ,SysJ,Cmp,sc] = OCC []  **

11         set SysM [Cmp,sc] = OCC []  **

11            cluster Mary [SysM,Cmp,sc] = OCC []  **

11               leafstate m1_IdleNoTok [Mary,SysM,Cmp,sc] = VAC []

11               leafstate m2_ReqdTok [Mary,SysM,Cmp,sc] = VAC []

11               leafstate m3_Waiting [Mary,SysM,Cmp,sc] = OCC []  **

11               leafstate m4_AllocPlain [Mary,SysM,Cmp,sc] = VAC []

11               leafstate m5_IdleWithTok [Mary,SysM,Cmp,sc] = VAC []

11               leafstate m6_AllocOtherWant [Mary,SysM,Cmp,sc] = VAC []

11            cluster UserM [SysM,Cmp,sc] = OCC []  **

11               leafstate NotHaveRes [UserM,SysM,Cmp,sc] = VAC []

11               leafstate Waiting [UserM,SysM,Cmp,sc] = OCC []  **

11               leafstate HaveRes [UserM,SysM,Cmp,sc] = VAC []

11   TRACE =[JAcq]

11   TREV [[JohnRelRes,[sc]],0,[],[ClientJohnPco,[sc]]]

11   TREV [[beta,[sc]],0,[],[UserJPco,[sc]]]

11   TREV [[JohnPass,[Cmp,sc]],0,[],[InterArbJohnPco,[Cmp,sc]]]

11   TREV [[MaryAcqRes,[sc]],0,[],[ClientMaryPco,[sc]]]

11   TREV [[GiveJohnTok,[sc]],0,[],[]]

outworlds=[11]

number of outworlds=1

SC:|: pe beta

SC:|: gc

17   statechart sc

17      set Cmp [sc] = OCC []  **

17         set SysJ [Cmp,sc] = OCC []  **

17            cluster John [SysJ,Cmp,sc] = OCC []  **

17               leafstate m1_IdleNoTok [John,SysJ,Cmp,sc] = OCC []  **

17               leafstate m2_ReqdTok [John,SysJ,Cmp,sc] = VAC []

17               leafstate m3_Waiting [John,SysJ,Cmp,sc] = VAC []

17               leafstate m4_AllocPlain [John,SysJ,Cmp,sc] = VAC []

17               leafstate m5_IdleWithTok [John,SysJ,Cmp,sc] = VAC []

17               leafstate m6_AllocOtherWant [John,SysJ,Cmp,sc] = VAC []

17            cluster UserJ [SysJ,Cmp,sc] = OCC []  **

17               leafstate NotHaveRes [UserJ,SysJ,Cmp,sc] = OCC []  **

17               leafstate Waiting [UserJ,SysJ,Cmp,sc] = VAC []

17               leafstate HaveRes [UserJ,SysJ,Cmp,sc] = VAC []

17         set SysM [Cmp,sc] = OCC []  **

17            cluster Mary [SysM,Cmp,sc] = OCC []  **

17               leafstate m1_IdleNoTok [Mary,SysM,Cmp,sc] = VAC []

17               leafstate m2_ReqdTok [Mary,SysM,Cmp,sc] = VAC []

17               leafstate m3_Waiting [Mary,SysM,Cmp,sc] = VAC []

17               leafstate m4_AllocPlain [Mary,SysM,Cmp,sc] = OCC []  **

17               leafstate m5_IdleWithTok [Mary,SysM,Cmp,sc] = VAC []

17               leafstate m6_AllocOtherWant [Mary,SysM,Cmp,sc] = VAC []

17            cluster UserM [SysM,Cmp,sc] = OCC []  **

17               leafstate NotHaveRes [UserM,SysM,Cmp,sc] = VAC []

17               leafstate Waiting [UserM,SysM,Cmp,sc] = VAC []

17               leafstate HaveRes [UserM,SysM,Cmp,sc] = OCC []  **

17   TRACE =[MAcq,JRel,JAcq]

17   TREV [[JohnReqRes,[sc]],0,[],[ClientJohnPco,[sc]]]

17   TREV [[alpha,[sc]],0,[],[UserJPco,[sc]]]

17   TREV [[MaryRelRes,[sc]],0,[],[ClientMaryPco,[sc]]]

17   TREV [[JohnReqTok,[Cmp,sc]],0,[],[InterArbJohnPco,[Cmp,sc]]]

17   TREV [[delta,[sc]],0,[],[UserMPco,[sc]]]

17   TREV [[GiveJohnTok,[sc]],0,[],[]]

outworlds=[17]

number of outworlds=1

SC:|: pe delta

SC:|: gc

20   statechart sc

20      set Cmp [sc] = OCC []  **

20         set SysJ [Cmp,sc] = OCC []  **

20            cluster John [SysJ,Cmp,sc] = OCC []  **

20               leafstate m1_IdleNoTok [John,SysJ,Cmp,sc] = OCC []  **

20               leafstate m2_ReqdTok [John,SysJ,Cmp,sc] = VAC []

20               leafstate m3_Waiting [John,SysJ,Cmp,sc] = VAC []

20               leafstate m4_AllocPlain [John,SysJ,Cmp,sc] = VAC []

20               leafstate m5_IdleWithTok [John,SysJ,Cmp,sc] = VAC []

20               leafstate m6_AllocOtherWant [John,SysJ,Cmp,sc] = VAC []

20            cluster UserJ [SysJ,Cmp,sc] = OCC []  **

20               leafstate NotHaveRes [UserJ,SysJ,Cmp,sc] = OCC []  **

20               leafstate Waiting [UserJ,SysJ,Cmp,sc] = VAC []

20               leafstate HaveRes [UserJ,SysJ,Cmp,sc] = VAC []

20         set SysM [Cmp,sc] = OCC []  **

20            cluster Mary [SysM,Cmp,sc] = OCC []  **

20               leafstate m1_IdleNoTok [Mary,SysM,Cmp,sc] = VAC []

20               leafstate m2_ReqdTok [Mary,SysM,Cmp,sc] = VAC []

20               leafstate m3_Waiting [Mary,SysM,Cmp,sc] = VAC []

20               leafstate m4_AllocPlain [Mary,SysM,Cmp,sc] = VAC []

20               leafstate m5_IdleWithTok [Mary,SysM,Cmp,sc] = OCC []  **

20               leafstate m6_AllocOtherWant [Mary,SysM,Cmp,sc] = VAC []

20            cluster UserM [SysM,Cmp,sc] = OCC []  **

20               leafstate NotHaveRes [UserM,SysM,Cmp,sc] = OCC []  **

20               leafstate Waiting [UserM,SysM,Cmp,sc] = VAC []

20               leafstate HaveRes [UserM,SysM,Cmp,sc] = VAC []

20   TRACE =[MRel,MAcq,JRel,JAcq]

20   TREV [[JohnReqRes,[sc]],0,[],[ClientJohnPco,[sc]]]

20   TREV [[alpha,[sc]],0,[],[UserJPco,[sc]]]

20   TREV [[JohnReqTok,[Cmp,sc]],0,[],[InterArbJohnPco,[Cmp,sc]]]

20   TREV [[MaryReqRes,[sc]],0,[],[ClientMaryPco,[sc]]]

20   TREV [[gamma,[sc]],0,[],[UserMPco,[sc]]]

20   TREV [[GiveJohnTok,[sc]],0,[],[]]

outworlds=[20]

number of outworlds=1

SC:|:

# The STATECRUNCHER distributed arbiter in CCS

The description of the model in Figure 8 is as follows, with the following renaming applied with respect to that figure:

MeReqRes → ReqRes

MeRelRes → RelRes

MeAcqRes → AcqRes

MeReqTok	→	ReqTok

YouReqTok 	→	R6e6q6T6o6k6

MePass	→	Pass

YouPass 	→	P6a6s6s6

MeSayOk	→	Ok

YouSayOk 	→	O6k6

MeSayNok	→	Nok

YouSayNok 	→	N6o6k6

Here is the description:

m1_IdleNoTok	=def	ReqRes.ReqTok.m2_ReqdTok

m2_ReqdTok	=def	N6o6k6.m3_Waiting + P6a6s6s6.AcqRes.m4_AllocPlain

m3_Waiting	=def	P6a6s6s6. AcqRes.m4_AllocPlain

m4_AllocPlain	=def	RelRes.m5_IdleWithTok + R6e6q6T6o6k6.Nok.m6_AllocOtherWant

m5_IdleWithTok=def	ReqRes.AcqRes.m4_AllocPlain+ R6e6q6T6o6k6.Pass.m1_IdleNoTok

m6_AllocOtherWant=def	RelRes.Pass.m1_IdleNoTok

We now consider the model given by Bruns.

I1

Idle

No Token

S1

Resource

Requested

n

ST1

Self Request

HaveToken

Dot

OG1/OA1

Other

Wants Token

Me

Req1

I request

the resource

Acq1

I acquire

the resource

__

Ok

I don't need the token

G1/A1

Resource

Allocated

Pass

You pass

the token

(to me)

___

Nok

I request

the token

(from you)

OT1

Token About to be Passed

IT1

Idle

WithToken

Nok

You request the token (from me)

Ok

You don't want the token (from me)

Rel1

I release

the resource

Nok

You request the token (from me)

____

Pass

I pass

the token

(to you)

No

Token

No

Token

Legend:   The following symbol represents having the token

(it is like a railway token)

Rel1

I release

the resource

- CCS Model in [Bruns]

Bruns's model is similar to the model in Figure 8. We make the following remarks

- Bruns has three minor typographical errors in his Figure 2.5:

- rel1 from state I1 should read req1

- state G1 (G for Got?) should read A1 (Allocated)

- state OG1 should read OA1

- The ok o4k4 event is initiated when one arbiter acquires the resource. It is pointless, because it checks for a request for the resource when there is no need to do so. An arbiter is told when there is request for a resource by the Nok event; any initiative taken in asking about a request is also expensive (because it involves inter-arbiter communication).

- The Nok N6o6k6 event is a request for the token. This will be triggered by a request for the resource when the arbiter does not have the token.

A comparison

We have named STATECRUNCHER states and events as seems natural in that language. The naming relationships between the STATECRUNCHER and Bruns's CCS model are:

- Bruns's 1 and 2 are replaced by Me and You for events, but we avoid such suffixes for states, which implicitly apply to the Me machine shown. When a second arbiter is introduced, states are by distinguished making names local to an arbiter.

- We distinguish YouPass and MePass as separate events

- We likewise distinguish who says Ok and Nok with YouSayOk, MeSayOk, YouSayNok and MeSayNok.

- Our m2_Reqd_Tok  and m3_Waiting states in Figure 8 are combined by Bruns into state S1 (resource requested).

We could merge the MePass and YouPass events into one event Pass, since no confusion would arise, but they are better considered as separate events. Similarly the other inter-arbiter events. They have separate origins - in separate computers even. In STATECRUNCHER it is convenient to give them separate names, since they can then be separately declared in their own machine, albeit with composition scope (through the use of scoping operators).

Bruns combines two arbiter agents with CCS calculus; STATECRUNCHER combines server and client by wrapping both in a set. STATECRUNCHER offers scoping operators for PCOs, events, states and variables so that these items can have local or composition scope (see the use of the %% operator in the arbiter-pair models).

Conclusion

This paper has shown how a typical client-server application is modelled in STATECRUNCHER, providing a direct comparison with a well-known example in the literature. Both STATECRUNCHER and CCS are amenable to the problem, but the emphasis is different: STATECRUNCHER is a state machine engine providing the white box or black box the oracle to tests and does not support calculus manipulations; CCS is a calculus which is used to prove properties of composed systems, and is supported by Concurrency Workbench.

# Source code of models

## Source code of the single distributed arbiter [model t4300]

//-----------------------------------------------------------------------------------

// Module:    d_arb.scs.txt

// Author:    Graham Thomason, Philips Digital Systems Laboratories, Redhill

// Date:      07 June, 2003

// Purpose:   Statecruncher model: SINGLE DISTRIBUTED ARBITER (cf Glenn Bruns CCS, p.21)

//

// Project:   Improving Component Integration

//

// Copyright (C) 2003 Philips Electronics N.V.

//

// Revision History:

//

//-------1---------2---------3---------4---------5---------6---------7---------8-----

statechart sc(Me)

PCO ClientMePco;        // For client-arbiter events

PCO InterArbMePco;      // For inter-arbiter events to Me

PCO InterArbYouPco;     // For inter-arbiter events to You

// no PCO for internal events

// Acq=Acquire

// Rel=Release

// Req=Request

// Res=Resource

// Tok=Token

// No need for the initial GiveMeTok event, because YouPass is legal

event MeReqRes  @ClientMePco;           // Client event to   Me

event MeRelRes  @ClientMePco;           // Client event to   Me

event MeAcqRes  @ClientMePco;           // Client event from Me

event MeReqTok  @InterArbMePco;         // InterArbiter

event MePass    @InterArbMePco;         // InterArbiter

event MeSayNok  @InterArbMePco;         // InterArbiter

event YouReqTok  @InterArbYouPco;       // InterArbiter

event YouPass    @InterArbYouPco;       // InterArbiter

event YouSayNok  @InterArbYouPco;       // InterArbiter

event TryTok;                           // Local to this arbiter

event TryOk;                            // Local to this arbiter

event AcqTok;                           // Local to this arbiter

event ResetWant;                        // Local to this arbiter

set Me(Res,Have,Need,OtherWant)

cluster Res(Idle,Requested,Waiting,Alloc)

state Idle      {MeReqRes->Requested {fire TryTok;}; }

state Requested {AcqTok,TryOk->Alloc; YouSayNok->Waiting;}

state Waiting   {AcqTok->Alloc;}

state Alloc     {upon enter {fire MeAcqRes;} MeRelRes->Idle             \

{if (in($$Me.OtherWant.OtherWantTok)) {fire MePass;}};}

cluster Have (NotHaveTok,HaveTok)

state NotHaveTok {                        \

TryTok {fire MeReqTok;};               \

YouPass->HaveTok {fire AcqTok;};       }

state HaveTok    {                        \

TryTok {fire TryOk;};                  \

MePass->NotHaveTok {fire ResetWant;};  }

cluster Need(NotNeedTok,NeedTok)

state NotNeedTok {                        \

YouReqTok {fire MePass;};              \

exit($$Me.Res.Idle)->NeedTok;          }

state NeedTok {                           \

YouReqTok {fire MeSayNok;};            \

enter($$Me.Res.Idle)->NotNeedTok;      }

cluster OtherWant(OtherNotWantTok,OtherWantTok)

state OtherNotWantTok {                                \

YouReqTok [    in($$Me.Need.NeedTok)                \

&& in($$Me.Have.HaveTok)]->OtherWantTok;}

state OtherWantTok    {ResetWant->OtherNotWantTok;}

//------------------------[end of module]--------------------------------------------

## Source code of  the two distributed arbiters  (John and Mary) [model t4301]

//-----------------------------------------------------------------------------------

// Module:    d_arb.scs.txt

// Author:    Graham Thomason, Philips Digital Systems Laboratories, Redhill

// Date:      07 June, 2003

// Purpose:   Statecruncher model: SINGLE DISTRIBUTED ARBITER (cf Glenn Bruns CCS, p.21)

//

// Project:   Improving Component Integration

//

// Copyright (C) 2003 Philips Electronics N.V.

//

// Revision History:

//

//-------1---------2---------3---------4---------5---------6---------7---------8-----

statechart sc(Me)

PCO ClientMePco;        // For client-arbiter events

PCO InterArbMePco;      // For inter-arbiter events to Me

PCO InterArbYouPco;     // For inter-arbiter events to You

// no PCO for internal events

// Acq=Acquire

// Rel=Release

// Req=Request

// Res=Resource

// Tok=Token

// No need for the initial GiveMeTok event, because YouPass is legal

event MeReqRes  @ClientMePco;           // Client event to   Me

event MeRelRes  @ClientMePco;           // Client event to   Me

event MeAcqRes  @ClientMePco;           // Client event from Me

event MeReqTok  @InterArbMePco;         // InterArbiter

event MePass    @InterArbMePco;         // InterArbiter

event MeSayNok  @InterArbMePco;         // InterArbiter

event YouReqTok  @InterArbYouPco;       // InterArbiter

event YouPass    @InterArbYouPco;       // InterArbiter

event YouSayNok  @InterArbYouPco;       // InterArbiter

event TryTok;                           // Local to this arbiter

event TryOk;                            // Local to this arbiter

event AcqTok;                           // Local to this arbiter

event ResetWant;                        // Local to this arbiter

set Me(Res,Have,Need,OtherWant)

cluster Res(Idle,Requested,Waiting,Alloc)

state Idle      {MeReqRes->Requested {fire TryTok;}; }

state Requested {AcqTok,TryOk->Alloc; YouSayNok->Waiting;}

state Waiting   {AcqTok->Alloc;}

state Alloc     {upon enter {fire MeAcqRes;} MeRelRes->Idle             \

{if (in($$Me.OtherWant.OtherWantTok)) {fire MePass;}};}

cluster Have (NotHaveTok,HaveTok)

state NotHaveTok {                        \

TryTok {fire MeReqTok;};               \

YouPass->HaveTok {fire AcqTok;};       }

state HaveTok    {                        \

TryTok {fire TryOk;};                  \

MePass->NotHaveTok {fire ResetWant;};  }

cluster Need(NotNeedTok,NeedTok)

state NotNeedTok {                        \

YouReqTok {fire MePass;};              \

exit($$Me.Res.Idle)->NeedTok;          }

state NeedTok {                           \

YouReqTok {fire MeSayNok;};            \

enter($$Me.Res.Idle)->NotNeedTok;      }

cluster OtherWant(OtherNotWantTok,OtherWantTok)

state OtherNotWantTok {                                \

YouReqTok [    in($$Me.Need.NeedTok)                \

&& in($$Me.Have.HaveTok)]->OtherWantTok;}

state OtherWantTok    {ResetWant->OtherNotWantTok;}

//------------------------[end of module]--------------------------------------------

## Source code of the distributed arbiter with clients [model t4302]

//-----------------------------------------------------------------------------------

// Module:    d_arb_client.scs.txt

// Author:    Graham Thomason, Philips Digital Systems Laboratories, Redhill

// Date:      07 June, 2003

// Purpose:   Statecruncher model: DISTRIBUTED ARBITER WITH CLIENTS

//             (as in Glenn Bruns CCS, p.21)

//

// Project:   Improving Component Integration

//

// Copyright (C) 2003 Philips Electronics N.V.

//

// Revision History:

//

//-------1---------2---------3---------4---------5---------6---------7---------8-----

statechart sc(Cmp)

PCO UserJPco;                     // For user events

PCO UserMPco;                     // For user events

PCO ClientJohnPco;                // For Client-to-arbiter events

PCO ClientMaryPco;                // For Client-to-arbiter events

// Cmp=Composition

// Acq=Acquire

// Rel=Release

// Req=Request

// Res=Resource

// Tok=Token

event alpha@UserJPco;                    // User event in UserJ

event beta @UserJPco;                    // User event in UserJ

event gamma@UserMPco;                    // User event in UserM

event delta@UserMPco;                    // User event in UserM

event JohnReqRes @ClientJohnPco;         // Client event   to John(arbiter)

event JohnRelRes @ClientJohnPco;         // Client event   to John(arbiter)

event JohnAcqRes @ClientJohnPco;         // Client event from John(arbiter)

event MaryReqRes @ClientMaryPco;         // Client event   to Mary(arbiter)

event MaryRelRes @ClientMaryPco;         // Client event   to Mary(arbiter)

event MaryAcqRes @ClientMaryPco;         // Client event from Mary(arbiter)

// INITIAL MANUAL EVENT TO BE GIVEN

//   It preserves symmetry between John/Mary

//      (otherwise reverse state order to get opposing default states)

//   It is GLOBAL for ease of entry

//      (we could have used event Composition%%JohnAcqTok)

event GiveJohnTok;

// ReqTok and AcqTok events are locally defined in composition scope

set Cmp(SysJ,SysM) {GiveJohnTok->Cmp.SysJ.John.Have.HaveTok;}

set SysJ(John,UserJ)

set John(Res,Have,Need,OtherWant)

PCO   Cmp%%InterArbJohnPco;               // For inter-arbiter events to John

event Cmp%%JohnReqTok@InterArbJohnPco;    // InterArbiter, Composition scope

event Cmp%%JohnPass@InterArbJohnPco;      // InterArbiter, Composition scope

event Cmp%%JohnSayNok@InterArbMaryPco;    // InterArbiter, Composition scope

event TryTok;           // Local to this arbiter

event TryOk;            // Local to this arbiter

event AcqTok;           // Local to this arbiter

event ResetWant;        // Local to this arbiter

cluster Res(Idle,Requested,Waiting,Alloc)

state Idle      {JohnReqRes->Requested {fire TryTok;}; }

state Requested {AcqTok,TryOk->Alloc; MarySayNok->Waiting;}

state Waiting   {AcqTok->Alloc;}

state Alloc     {upon enter {fire JohnAcqRes;} JohnRelRes->Idle             \

{if (in($$John.OtherWant.OtherWantTok)) {fire JohnPass;}};}

cluster Have (NotHaveTok,HaveTok)

state NotHaveTok {                          \

TryTok {fire JohnReqTok;};               \

MaryPass->HaveTok {fire AcqTok;};        }

state HaveTok    {                          \

TryTok {fire TryOk;};                    \

JohnPass->NotHaveTok {fire ResetWant;};  }

cluster Need(NotNeedTok,NeedTok)

state NotNeedTok {                          \

MaryReqTok {fire JohnPass;};             \

exit($$John.Res.Idle)->NeedTok;          }

state NeedTok {                             \

MaryReqTok {fire JohnSayNok;};           \

enter($$John.Res.Idle)->NotNeedTok;      }

cluster OtherWant(OtherNotWantTok,OtherWantTok)

state OtherNotWantTok {                                  \

MaryReqTok [   in($$John.Need.NeedTok)                \

&& in($$John.Have.HaveTok)]->OtherWantTok;}

state OtherWantTok    {ResetWant->OtherNotWantTok;}

cluster UserJ(NotHaveRes,Waiting,HaveRes)

state NotHaveRes  {alpha->Waiting {fire JohnReqRes;};}

state Waiting     {JohnAcqRes->HaveRes;}

state HaveRes     {upon enter {trace("JAcq");}         \

upon exit  {trace("JRel");}         \

beta->NotHaveRes {fire JohnRelRes;};}

//----- as above except alpha,beta,gamma,delta and...

//                     ... Mary for John and vice versa everywhere -----

set SysM(Mary,UserM)

set Mary(Res,Have,Need,OtherWant)

PCO   Cmp%%InterArbMaryPco;               // For inter-arbiter events to Mary

event Cmp%%MaryReqTok @InterArbMaryPco;   // InterArbiter, Composition scope

event Cmp%%MaryPass   @InterArbMaryPco;   // InterArbiter, Composition scope

event Cmp%%MarySayNok @InterArbJohnPco;   // InterArbiter, Composition scope

event TryTok;           // Local to this arbiter

event TryOk;            // Local to this arbiter

event AcqTok;           // Local to this arbiter

event ResetWant;        // Local to this arbiter

cluster Res(Idle,Requested,Waiting,Alloc)

state Idle      {MaryReqRes->Requested {fire TryTok;}; }

state Requested {AcqTok,TryOk->Alloc; JohnSayNok->Waiting;}

state Waiting   {AcqTok->Alloc;}

state Alloc     {upon enter {fire MaryAcqRes;} MaryRelRes->Idle             \

{if (in($$Mary.OtherWant.OtherWantTok)) {fire MaryPass;}};}

cluster Have (NotHaveTok,HaveTok)

state NotHaveTok {                          \

TryTok {fire MaryReqTok;};               \

JohnPass->HaveTok {fire AcqTok;};        }

state HaveTok    {                          \

TryTok {fire TryOk;};                    \

MaryPass->NotHaveTok {fire ResetWant;};  }

cluster Need(NotNeedTok,NeedTok)

state NotNeedTok {                          \

JohnReqTok {fire MaryPass;};             \

exit($$Mary.Res.Idle)->NeedTok;          }

state NeedTok {                             \

JohnReqTok {fire MarySayNok;};           \

enter($$Mary.Res.Idle)->NotNeedTok;      }

cluster OtherWant(OtherNotWantTok,OtherWantTok)

state OtherNotWantTok {                                  \

JohnReqTok [   in($$Mary.Need.NeedTok)                \

&& in($$Mary.Have.HaveTok)]->OtherWantTok;}

state OtherWantTok    {ResetWant->OtherNotWantTok;}

cluster UserM(NotHaveRes,Waiting,HaveRes)

state NotHaveRes  {gamma->Waiting {fire MaryReqRes;};}

state Waiting     {MaryAcqRes->HaveRes;}

state HaveRes     {upon enter {trace("MAcq");}          \

upon exit  {trace("MRel");}          \

delta->NotHaveRes {fire MaryRelRes;};}

//------------------------[end of module]--------------------------------------------

## Source code of single flattened distributed arbiter [model t4310]

//-----------------------------------------------------------------------------------

// Module:    d_arbf.scs.txt

// Author:    Graham Thomason, Philips Digital Systems Laboratories, Redhill

// Date:      25 June, 2003

// Purpose:   Statecruncher model: SINGLE DISTRIBUTED ARBITER FLAT MODEL

//                                (cf Glenn Bruns CCS, p.21)

//

// Project:   Improving Component Integration

//

// Copyright (C) 2003 Philips Electronics N.V.

//-------1---------2---------3---------4---------5---------6---------7---------8-----

statechart sc(Me)

PCO ClientMePco;          // For client-arbiter events

PCO InterArbMePco;        // For inter-arbiter events to Me

PCO InterArbYouPco;       // For inter-arbiter events to You

// no PCO for internal events

// Acq=Acquire  (not used in flattened model)

// Rel=Release

// Req=Request

// Res=Resource

// Tok=Token

event MeReqRes  @ClientMePco;         // Client event to   Me

event MeRelRes  @ClientMePco;         // Client event to   Me

event MeAcqRes  @ClientMePco;         // Client event from Me

event MeReqTok @InterArbMePco;        // InterArbiter

event MePass   @InterArbMePco;        // InterArbiter

event MeSayNok @InterArbMePco;        // InterArbiter

event YouReqTok @InterArbYouPco;      // InterArbiter

event YouPass   @InterArbYouPco;      // InterArbiter

event YouSayNok @InterArbYouPco;      // InterArbiter

// No local events in the flattened model

cluster Me(  m1_IdleNoTok,   m2_ReqdTok,         \

m3_Waiting,     m4_AllocPlain,    \

m5_IdleWithTok, m6_AllocOtherWant )

state m1_IdleNoTok      { MeReqRes->m2_ReqdTok           {fire MeReqTok;}; }

state m2_ReqdTok        { YouSayNok->m3_Waiting;  \

YouPass->m4_AllocPlain; }

state m3_Waiting        { YouPass->m4_AllocPlain;}

state m4_AllocPlain     { upon enter {fire MeAcqRes;}                      \

MeRelRes->m5_IdleWithTok;                      \

YouReqTok->m6_AllocOtherWant {fire MeSayNok;}; }

state m5_IdleWithTok    { YouReqTok->m1_IdleNoTok        {fire MePass;};   \

MeReqRes->m4_AllocPlain;                         }

state m6_AllocOtherWant { MeRelRes->m1_IdleNoTok         {fire MePass;};   }

//------------------------[end of module]--------------------------------------------

## Source code of flattened distributed arbiters [model t4311]

//-----------------------------------------------------------------------------------

// Module:    d_arbf_pair.scs.txt

// Author:    Graham Thomason, Philips Digital Systems Laboratories, Redhill

// Date:      25 June, 2003

// Purpose:   Statecruncher model: TWO DISTRIBUTED ARBITERS FLATTENED

//                                (cf Glenn Bruns CCS, p.21)

//

// Project:   Improving Component Integration

//

// Copyright (C) 2003 Philips Electronics N.V.

//

// Revision History:

//

//-------1---------2---------3---------4---------5---------6---------7---------8-----

statechart sc(Cmp)

PCO ClientJohnPco;         // For user-to-arbiter events

PCO ClientMaryPco;         // For user-to-arbiter events

// Acq=Acquire  (not used in flattened model)

// Rel=Release

// Req=Request

// Res=Resource

// Tok=Token

event JohnReqRes @ClientJohnPco;          // Client event   to John(arbiter)

event JohnRelRes @ClientJohnPco;          // Client event   to John(arbiter)

event JohnAcqRes @ClientJohnPco;          // Client event from John(arbiter)

event MaryReqRes @ClientMaryPco;          // Client event   to Mary(arbiter)

event MaryRelRes @ClientMaryPco;          // Client event   to Mary(arbiter)

event MaryAcqRes @ClientMaryPco;          // Client event from Mary(arbiter)

// No local events in the flattened model

// INITIAL MANUAL EVENT TO BE GIVEN

event GiveJohnTok;

set Cmp(John,Mary) {                                                       \

GiveJohnTok[in(Cmp.John.m1_IdleNoTok) && in(Cmp.Mary.m1_IdleNoTok)] \

->Cmp.John.m5_IdleWithTok;                                       }

// The above condition is evaluated at execution time and does not

// prevent the event appearing as a potential transitionable event

cluster John(  m1_IdleNoTok,   m2_ReqdTok,       \

m3_Waiting,     m4_AllocPlain,    \

m5_IdleWithTok, m6_AllocOtherWant )

PCO   Cmp%%InterArbJohnPco;               // For inter-arbiter events to John

event Cmp%%JohnReqTok @InterArbJohnPco;   // InterArbiter, Composition scope

event Cmp%%JohnPass   @InterArbJohnPco;   // InterArbiter, Composition scope

event Cmp%%JohnSayNok @InterArbMaryPco;   // InterArbiter, Composition scope

state m1_IdleNoTok      { JohnReqRes->m2_ReqdTok        {fire JohnReqTok;}; }

state m2_ReqdTok        { MarySayNok->m3_Waiting;  \

MaryPass->m4_AllocPlain; }

state m3_Waiting        { MaryPass->m4_AllocPlain;}

state m4_AllocPlain     { upon enter {fire JohnAcqRes;}                       \

JohnRelRes->m5_IdleWithTok;                       \

MaryReqTok->m6_AllocOtherWant {fire JohnSayNok;}; }

state m5_IdleWithTok    { MaryReqTok->m1_IdleNoTok      {fire JohnPass;};   \

JohnReqRes->m4_AllocPlain;                        }

state m6_AllocOtherWant { JohnRelRes->m1_IdleNoTok      {fire JohnPass;};   }

//----- as above, but Mary for John and vice versa everywhere -----

cluster Mary(  m1_IdleNoTok,   m2_ReqdTok,       \

m3_Waiting,     m4_AllocPlain,    \

m5_IdleWithTok, m6_AllocOtherWant )

PCO   Cmp%%InterArbMaryPco;               // For inter-arbiter events to Mary

event Cmp%%MaryReqTok @InterArbMaryPco;   // InterArbiter, Composition scope

event Cmp%%MaryPass   @InterArbMaryPco;   // InterArbiter, Composition scope

event Cmp%%MarySayNok @InterArbJohnPco;   // InterArbiter, Composition scope

state m1_IdleNoTok      { MaryReqRes->m2_ReqdTok        {fire MaryReqTok;}; }

state m2_ReqdTok        { JohnSayNok->m3_Waiting;  \

JohnPass->m4_AllocPlain; }

state m3_Waiting        { JohnPass->m4_AllocPlain;}

state m4_AllocPlain     { upon enter {fire MaryAcqRes;}                       \

MaryRelRes->m5_IdleWithTok;                       \

JohnReqTok->m6_AllocOtherWant {fire MarySayNok;}; }

state m5_IdleWithTok    { JohnReqTok->m1_IdleNoTok      {fire MaryPass;};   \

MaryReqRes->m4_AllocPlain;                        }

state m6_AllocOtherWant { MaryRelRes->m1_IdleNoTok      {fire MaryPass;};   }

//------------------------[end of module]--------------------------------------------

## Source code of flattened distributed arbiter with clients [model t4312]

//-----------------------------------------------------------------------------------

// Module:    d_arbf_client.scs.txt

// Author:    Graham Thomason, Philips Digital Systems Laboratories, Redhill

// Date:      25 June, 2003

// Purpose:   Statecruncher model: DISTRIBUTED (FLAT) ARBITER WITH CLIENTS

//                                (cf Glenn Bruns CCS, p.21)

//

// Project:   Improving Component Integration

//

// Copyright (C) 2003 Philips Electronics N.V.

//

// Revision History:

//

//-------1---------2---------3---------4---------5---------6---------7---------8-----

statechart sc(Cmp)

PCO UserJPco;                     // For user events

PCO UserMPco;                     // For user events

PCO ClientJohnPco;                // For user-to-arbiter events

PCO ClientMaryPco;                // For user-to-arbiter events

// Acq=Acquire  (not used in flattened model)

// Rel=Release

// Req=Request

// Res=Resource

// Tok=Token

event alpha@UserJPco;                    // User event in UserJ

event beta @UserJPco;                    // User event in UserJ

event gamma@UserMPco;                    // User event in UserM

event delta@UserMPco;                    // User event in UserM

event JohnReqRes @ClientJohnPco;         // Client event   to John(arbiter)

event JohnRelRes @ClientJohnPco;         // Client event   to John(arbiter)

event JohnAcqRes @ClientJohnPco;         // Client event from John(arbiter)

event MaryReqRes @ClientMaryPco;         // Client event   to Mary(arbiter)

event MaryRelRes @ClientMaryPco;         // Client event   to Mary(arbiter)

event MaryAcqRes @ClientMaryPco;         // Client event from Mary(arbiter)

// No local events in the flattened model

// INITIAL MANUAL EVENT TO BE GIVEN

event GiveJohnTok;

set Cmp(SysJ,SysM) { GiveJohnTok                                               \

[in(Cmp.SysJ.John.m1_IdleNoTok) && in(Cmp.SysM.Mary.m1_IdleNoTok)] \

->Cmp.SysJ.John.m5_IdleWithTok;                                    }

// The above condition is evaluated at execution time and does not

// prevent the event appearing as a potential transitionable event

set SysJ(John,UserJ)

cluster John(  m1_IdleNoTok,   m2_ReqdTok,       \

m3_Waiting,     m4_AllocPlain,    \

m5_IdleWithTok, m6_AllocOtherWant )

PCO   Cmp%%InterArbJohnPco;               // For inter-arbiter events to John

event Cmp%%JohnReqTok @InterArbJohnPco;   // InterArbiter, Composition scope

event Cmp%%JohnPass   @InterArbJohnPco;   // InterArbiter, Composition scope

event Cmp%%JohnSayNok @InterArbMaryPco;   // InterArbiter, Composition scope

state m1_IdleNoTok      { JohnReqRes->m2_ReqdTok        {fire JohnReqTok;}; }

state m2_ReqdTok        { MarySayNok->m3_Waiting;  \

MaryPass->m4_AllocPlain; }

state m3_Waiting        { MaryPass->m4_AllocPlain;}

state m4_AllocPlain     { upon enter {fire JohnAcqRes;}                       \

JohnRelRes->m5_IdleWithTok;                       \

MaryReqTok->m6_AllocOtherWant {fire JohnSayNok;}; }

state m5_IdleWithTok    { MaryReqTok->m1_IdleNoTok      {fire JohnPass;};   \

JohnReqRes->m4_AllocPlain;                        }

state m6_AllocOtherWant { JohnRelRes->m1_IdleNoTok      {fire JohnPass;};   }

cluster UserJ(NotHaveRes,Waiting,HaveRes)

state NotHaveRes  {alpha->Waiting {fire JohnReqRes;};}

state Waiting     {JohnAcqRes->HaveRes;}

state HaveRes     {upon enter {trace("JAcq");}         \

upon exit  {trace("JRel");}         \

beta->NotHaveRes {fire JohnRelRes;};}

//----- as above, but Mary for John and vice versa everywhere -----

set SysM(Mary,UserM)

cluster Mary(  m1_IdleNoTok,   m2_ReqdTok,       \

m3_Waiting,     m4_AllocPlain,    \

m5_IdleWithTok, m6_AllocOtherWant )

PCO   Cmp%%InterArbMaryPco;               // For inter-arbiter events to Mary

event Cmp%%MaryReqTok @InterArbMaryPco;   // InterArbiter, Composition scope

event Cmp%%MaryPass   @InterArbMaryPco;   // InterArbiter, Composition scope

event Cmp%%MarySayNok @InterArbJohnPco;   // InterArbiter, Composition scope

state m1_IdleNoTok      { MaryReqRes->m2_ReqdTok        {fire MaryReqTok;}; }

state m2_ReqdTok        { JohnSayNok->m3_Waiting;  \

JohnPass->m4_AllocPlain; }

state m3_Waiting        { JohnPass->m4_AllocPlain;}

state m4_AllocPlain     { upon enter {fire MaryAcqRes;}                       \

MaryRelRes->m5_IdleWithTok;                       \

JohnReqTok->m6_AllocOtherWant {fire MarySayNok;}; }

state m5_IdleWithTok    { JohnReqTok->m1_IdleNoTok      {fire MaryPass;};   \

MaryReqRes->m4_AllocPlain;                        }

state m6_AllocOtherWant { MaryRelRes->m1_IdleNoTok      {fire MaryPass;};   }

cluster UserM(NotHaveRes,Waiting,HaveRes)

state NotHaveRes  {gamma->Waiting {fire MaryReqRes;};}

state Waiting     {MaryAcqRes->HaveRes;}

state HaveRes     {upon enter {trace("MAcq");}          \

upon exit  {trace("MRel");}          \

delta->NotHaveRes {fire MaryRelRes;};}

//------------------------[end of module]--------------------------------------------

# References

STATECRUNCHER documentation and papers by the present author

| Main Thesis | [StCrMain] | The Design and Construction of a State Machine System that Handles Nondeterminism |
| --- | --- | --- |
| Appendices |  |  |
| Appendix 1 | [StCrContext] | Software Testing in Context |
| Appendix 2 | [StCrSemComp] | A Semantic Comparison of STATECRUNCHER and Process Algebras |
| Appendix 3 | [StCrOutput] | A Quick Reference of STATECRUNCHER's Output Format |
| Appendix 4 | [StCrDistArb] | Distributed Arbiter Modelling in CCS and STATECRUNCHER - A Comparison |
| Appendix 5 | [StCrNim] | The Game of Nim in Z and STATECRUNCHER |
| Appendix 6 | [StCrBiblRef] | Bibliography and References |
| Related reports |  |  |
| Related report 1 | [StCrPrimer] | STATECRUNCHER-to-Primer Protocol |
| Related report 2 | [StCrManual] | STATECRUNCHER User Manual |
| Related report 3 | [StCrGP4] | GP4 - The Generic Prolog Parsing and Prototyping Package (underlies the STATECRUNCHER compiler) |
| Related report 4 | [StCrParsing] | STATECRUNCHER Parsing |
| Related report 5 | [StCrTest] | STATECRUNCHER Test Models |
| Related report 6 | [StCrFunMod] | State-based Modelling of Functions and Pump Engines |

References

[Bruns]	Glenn Bruns

Distributed Systems Analysis with CCS

Prentice Hall 1997, ISBN 0-13-398389-7

[CWB]	The Edinburgh Concurrency Workbench

http://www.dcs.ed.ac.uk/home/cwb/

[Harel 87]	D. Harel et al.

On the Formal Semantics of Statecharts

Logic in Computer Science, 2nd Annual Conference, 1987, pp.54-64

[Milner]	Robin Milner

Communication and Concurrency

Prentice Hall 1997, ISBN 0-13-114984-9 and 0-13-115007-3 Pbk

[UML]	The Object Management Group website is: http://www.omg.org

UML specifications are available from this website.